Department of Computer Science and Engineering

Assignment Questions

Course Code & Name :  CSA1223- Computer Architecture

| Assignment Title: | Cache Mapping and Replacement Policy Co-Design: Performance, Memory and CPU Analysis |
| --- | --- |
| Course Outcome(s) – CO: | CO1-Analyze the functional units of a computer system including CPU, memory, bus structures, and I/O components by examining instruction execution cycles, addressing modes, and performance metrics such as CPI, clock cycles, and benchmarking CO2-Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems CO3-Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT) CO4-Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics CO5-Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt |
| Bloom’s Taxonomy Level | L3 – Apply; L4 – Analyse, L5 – Evaluate |
| SDG Mapping (SDG 1-17) | SDG 11 – Sustainable Cities and Communities; SDG 12 – Responsible Consumption and Production |

Problem Statement : A high-performance edge workstation must execute a memory-intensive analytics workload. The system needs a cache organization that reduces conflict misses while keeping latency, memory traffic and hardware complexity within practical limits.

Design the cache hierarchy, determine address-mapping fields, compare direct-mapped and set-associative organizations, evaluate LRU and FIFO replacement policies, and simulate the workload using Python. Generate controlled sequential, repeated and conflict-heavy memory accesses and evaluate cache hit rate, miss rate, effective CPI, execution time and memory traffic. Compare the alternatives and recommend the better cache design based on performance and practical constraints.

ASSIGNMENT PROBLEM / CHALLENGE

Industry Context

A technology company is developing a compact edge-computing workstation for real-time analytics. The application repeatedly accesses feature vectors, lookup tables and model parameters stored in main memory.

The workload contains sequential accesses, repeated hot-set accesses and conflict-heavy references. Therefore, overall system performance depends strongly on cache mapping, associativity, replacement policy and memory latency.

The system must maintain low execution latency while minimizing cache misses and unnecessary memory traffic.

PROBLEM STATEMENT

A technology company is designing a compact analytics workstation. The processor operates at 2.4 GHz with an ideal CPI of 1.0 and executes a representative workload of 1.2 million instructions.

The workload contains the following instruction categories:

Instruction Mix = Integer ALU + Memory + Floating-point + Branch + Control/Other

= 40% + 25% + 15% + 10% + 10%

= 100%

The application executes:

- Integer ALU operations

- Memory load/store operations

- Floating-point operations

- Branching

- Control/Other instructions

The baseline processor operates at 2.4 GHz with an ideal CPI of 1.0.

A representative workload contains 1.2 million instructions, with the following instruction distribution:

| Instruction Category | Percentage | Number of Instructions |
| --- | --- | --- |
| Integer ALU | 40% | 480,000 |
| Memory | 25% | 300,000 |
| Floating-point | 15% | 180,000 |
| Branch | 10% | 120,000 |
| Control/Other | 10% | 120,000 |
| Total | 100% | 1,200,000 |

Alternative A

A direct-mapped 4 KB L1 cache with 64-byte cache lines and simple lookup hardware.

Alternative B

A 4-way set-associative 4 KB L1 cache using LRU replacement, providing more resistance to conflict misses.

Both alternatives must correctly support the complete instruction workload and reliable movement of cache lines between the processor and main memory.

The architectures must be evaluated using controlled sequential, repeated and conflict-heavy memory references.

The following performance parameters must be quantified:

- Cache hit rate

- Cache miss rate

- Effective CPI

- Total execution cycles

- Execution time

- Cache misses

- Conflict-miss behaviour

- Memory-line fills

- Memory traffic

- Replacement-policy behaviour

- Hardware complexity

The final architecture must satisfy both of the following application targets:

1. Average execution time < 1.1 ms per workload

2. Cache hit rate ≥ 90%

REQUIREMENTS AND CONSTRAINTS

Functional Requirements

- The processor shall execute integer ALU operations.

- The processor shall support memory load/store operations through the cache hierarchy.

- The processor shall correctly execute floating-point operations.

- The processor shall correctly execute branch and control instructions.

- The cache shall store and retrieve 64-byte cache lines.

- The memory subsystem shall support sequential, repeated and conflict-heavy access patterns.

- The architecture shall provide reliable movement of data between the processor, cache and main memory.

- Performance Requirements

| Parameter | Required Value |
| --- | --- |
| Processor frequency | 2.4 GHz |
| Ideal CPI | 1.0 |
| Workload size | 1,200,000 instructions |
| L1 cache capacity | 4 KB |
| Cache line size | 64 bytes |
| Address space | 64 KB |
| Target cache hit rate | ≥ 90% |

Workload Requirements

The simulation shall use the instruction distribution specified in the problem:

- 40% Integer ALU

- 25% Memory

- 15% Floating-point

- 10% Branch

- 10% Control/Other

The memory workload shall include:

- Sequential memory accesses

- Repeated hot-set accesses

- Conflict-heavy accesses

Cache Analysis Requirements

The analysis shall identify and quantify:

- Cache hits

- Cache misses

- Conflict misses

- Memory-line fills

Alternative A shall be analysed as a direct-mapped cache, while Alternative B shall be analysed as a 4-way set-associative cache using LRU replacement.

Memory Requirements

The architecture shall include a 4 KB L1 cache suitable for the analytics workload.

The analysis shall consider:

- Cache accesses

- Cache hits

- Controlled cache misses

- Cache-miss penalties

- Memory-access latency

- Memory traffic

- Effect of cache behaviour on CPI and execution time

- Replacement Policy Requirements

- The system shall compare LRU and FIFO replacement for set-associative configurations.

- The cache-line size shall be:

- 64 bytes

- The model shall also evaluate the resulting memory traffic and execution-time impact.

Simulation Requirements

## The implementation shall produce quantitative results for:

- Cache hit rate

- Cache misses

- Effective CPI

- Execution cycles

- Execution time

- Memory-line fills

- Memory traffic

- Replacement-policy behaviour

- Inference/workload latency

## The final results shall be used to determine whether each cache architecture satisfies the specified latency and hit-rate requirements.

## Engineering Constraints

## The final decision shall consider the following:

- Performance

- Hardware complexity

- Resource utilization

- Scalability

- Memory traffic

- Cache implementation limitations

- Efficient use of computing resources

## STUDENT WORK

## PROBLEM UNDERSTANDING

## The given system is a compact analytics workstation that must execute a memory-intensive workload at 2.4 GHz. Because main memory is much slower than the processor, cache misses can add a significant number of stall cycles.

## The workload contains integer ALU, memory, floating-point, branch and control/other instructions. The memory references are deliberately structured to expose the effect of cache mapping and replacement policy.

## The main challenge is to determine which cache organization can maintain a hit rate of at least 90% and execution time below 1.1 ms while avoiding unnecessary hardware complexity.

## The analysis focuses on the interaction between:

- Processor pipeline

- L1 cache organization

- Address mapping

- Associativity

- Replacement policy

- Main memory

- Cache-line fills

- Effective CPI

## Available Information

| ## Parameter | ## Given Value |
| --- | --- |
| Processor frequency | 2.4 GHz |
| Ideal CPI | 1.0 |
| Workload size | 1,200,000 instructions |
| L1 cache capacity | 4 KB |
| Cache line size | 64 bytes |
| Address space | 64 KB |
| Integer ALU instructions | 40% |
| Memory instructions | 25% |
| Floating-point instructions | 15% |
| Branch instructions | 10% |
| Control/Other instructions | 10% |
| Required average execution time | < 1.1 ms |

## Workload Formulation

## The representative workload contains exactly 1,200,000 instructions.

| ## Instruction Type | ## Count |
| --- | --- |
| Integer ALU | 480,000 |
| Memory | 300,000 |
| Floating-point | 180,000 |
| Branch | 120,000 |
| Control/Other | 120,000 |
| Total | 1,200,000 |

## BASELINE PERFORMANCE ANALYSIS

## The baseline processor operates at 2.4 GHz and has an ideal CPI of 1.0.

## The ideal number of processor cycles is:

## Ideal cycles = 1,200,000 × 1.0 = 1,200,000 cycles

## The processor clock period is:

## Clock period = 1 / 2.4 GHz ≈ 0.4167 ns

## The ideal execution time is:

## Ideal time = 1,200,000 / 2.4 × 10⁹ = 0.500 ms

## Thus, under ideal conditions, the workload requires approximately 0.500 ms.

## Cache Capacity Analysis

## Cache capacity = 4 KB and cache-line size = 64 bytes.

## 12 MB/s

## Number of cache lines = 4096 / 64 = 64 lines

## Bandwidthaggregate=4×12
Bandwidthaggregate=48 MB/s

## Therefore, the direct-mapped cache has 64 sets, while a 4-way cache has 16 sets with four lines per set.

## Success Criteria

## The proposed architecture will be considered successful only when both mandatory requirements are satisfied:

## Average Inference Latency<2.0 ms

## Execution time < 1.1 ms

## Aggregate Input Bandwidth≥48 MB/s

## In addition, the final recommendation will consider hit rate, miss count, CPI, memory traffic, replacement behaviour, complexity, scalability and resource efficiency.

## Solution / Design / Methodology

## Alternative A – Direct-Mapped Cache

- 4 KB L1 cache with 64-byte lines and 64 sets

- One cache line per set

- Simple tag comparison and low hardware complexity

- Conflict misses can be high when multiple blocks map to the same set

- Cache misses fetch complete 64-byte lines from main memory

- Suitable where simplicity and low implementation cost are priorities

## Alternative B – 4-Way Set-Associative Cache

- 4 KB L1 cache with 64-byte lines and 16 sets

- Four cache lines can coexist in each set

- LRU replacement retains recently used blocks

- Reduced conflict misses for repeated hot-set accesses

- Higher comparison and replacement hardware complexity

- Higher performance for the controlled conflict-heavy workload

## Data Flow

## Main Memory → Cache → Processor

## Workload Model

- Integer ALU: 40%

- Memory: 25%

- Floating-point: 15%

- Branches: 10%

- Control/Other: 10%

## Use of Modern Tools

## Tools Used:

- Python – cache simulation, address mapping and performance calculations

## FREQ = 2.4e9

## N = 1_200_000

## MEM_REFS = 300_000

## CACHE_SIZE = 4096

## LINE_SIZE = 64

## MISS_PENALTY = 30

## def make_trace():

## trace = []

## for i in range(100_000):

## trace.append((i % 64) * LINE_SIZE)

## hot = [0, 64*64, 128*64, 192*64]

## for i in range(100_000):

## trace.append(hot[i % 4])

## pattern = [0, 64*64, 0, 128*64]

## for i in range(100_000):

## trace.append(pattern[i % 4])

## return trace

## def simulate(trace, ways, policy="LRU"):

## sets = (CACHE_SIZE // LINE_SIZE) // ways

## cache = [deque(maxlen=ways) for _ in range(sets)]

## hits = misses = 0

## for addr in trace:

## block = addr // LINE_SIZE

## index = block % sets

## current = cache[index]

## if block in current:

## hits += 1

## if policy == "LRU":

## current.remove(block)

## current.append(block)

## else:

## misses += 1

## if len(current) == ways:

## current.popleft()

## current.append(block)

## return hits, misses

## trace = make_trace()

## configs = [

## ("Direct", 1, "LRU"),

## ("2-way LRU", 2, "LRU"),

## ("4-way LRU", 4, "LRU"),

## ("2-way FIFO", 2, "FIFO"),

## ("4-way FIFO", 4, "FIFO")

## ]

## for name, ways, policy in configs:

## hits, misses = simulate(trace, ways, policy)

## hit_rate = hits / len(trace)

## cycles = N + misses * MISS_PENALTY

## cpi = cycles / N

## latency = cycles / FREQ * 1000

## traffic_kb = misses * LINE_SIZE / 1024

## print(name)

## print("Hits:", hits)

## print("Misses:", misses)

## print("Hit rate:", round(hit_rate, 4))

## print("CPI:", round(cpi, 4))

## print("Latency:", round(latency, 4), "ms")

## print("Memory traffic:", round(traffic_kb, 2), "KB")

Results and Validation

| Metric | Direct-Mapped | 4-way LRU |
| --- | --- | --- |
| Total Cycles | 7,200,760 | 1,200,200 |
| CPI | 6.0006 | 1.0002 |
| Execution Time | 3.0003 ms | 0.5001 ms |
| Cache Misses | 200,063 | 67 |
| Hit Rate | 33.31% | 99.98% |
| Memory Traffic | 12,503.94 KB | 4.19 KB |
| Latency Requirement | FAIL | PASS |
| Hit-rate Requirement | FAIL | PASS |

Validation: The 4-way LRU configuration satisfies the required ≥ 90% hit rate and < 1.1 ms execution-time targets under the stated simulation assumptions. The direct-mapped design fails both targets because of repeated conflict misses.

Analysis and Engineering Decision

Comparison: Alternative B provides better performance because four blocks can coexist in a set, greatly reducing conflict evictions for the repeated hot-set workload.

Decision: The 4-way LRU cache is recommended because it achieves the highest simulated hit rate and the lowest execution time among the evaluated configurations.

Trade-off: Alternative B requires more tag comparisons and replacement-state hardware than the direct-mapped cache.

Broader Considerations

- Sustainability: Reducing cache misses can reduce unnecessary memory-line transfers.

- Resource Usage: Direct mapping requires less hardware, while higher associativity uses additional comparison and replacement resources.

- Scalability: Set-associative caches are better suited to workloads with repeated and conflict-heavy access patterns.

- Professional Responsibility: Performance claims should be based on repeatable traces and clearly stated simulation assumptions.

### Conclusion

- The analysis compared direct-mapped and set-associative cache organizations using controlled memory-access traces.

- The 4-way LRU cache achieved the best simulated hit rate and execution time while satisfying the specified performance targets.

- The 4-way LRU design is therefore selected as the preferred architecture, with additional cache-control complexity as the main trade-off.

References

- Course notes and lecture materials – CSA1223: Computer Architecture.

- Faculty-provided CSA1223 Assignment Problem Statement.

- Faculty-provided Common Course Assignment Format – FINAL.

- Python documentation – used for simulation implementation.

- Computer Organization and Design – cache and memory hierarchy concepts.

- Common Assessment Rubric

| Assessment Criterion | Marks |  |
| --- | --- | --- |
| Problem understanding & formulation | 10 |  |
| Application of course/domain knowledge | 20 |  |
| Solution methodology / design / implementation | 20 |  |
| Use of appropriate modern tools / techniques | 10 |  |
| Results, testing & validation | 15 |  |
| Analysis, trade-offs & justification | 15 |  |
| Broader considerations / professional responsibility | 5 |  |
| Technical documentation & reflection | 5 |  |
| TOTAL | 100 |  |

- CO–PO–Assessment Mapping

| Assessment Component | CO | PO(s) | Bloom's Level | Marks |
| --- | --- | --- | --- | --- |
| Problem formulation | CO3, CO8 | PO2, PO3 | L4 | 10 |
| Application of knowledge | CO3, CO4, CO5 | PO1, PO2 | L3/L4 | 20 |
| Solution / design / implementation | CO3, CO4, CO7 | PO3, PO5 | L4/L5/L6 | 20 |
| Modern tool usage | CO7, CO8 | PO5 | L3/L4 | 10 |
| Validation | CO4, CO7 | PO2, PO5 | L4/L5 | 15 |
| Analysis & justification | CO7, CO8 | PO2, PO4 | L4/L5 | 15 |
| Broader considerations | CO8 | PO6, PO7 | L4/L5 | 5 |
| Documentation & reflection | CO6, CO8 | PO10 | L3/L4 | 5 |

Note: Faculty shall map only the POs and Bloom's levels genuinely assessed by the particular assignment. Every assignment is not required to map to every PO.

Faculty Evaluation & Continuous Improvement CO Attainment

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment:

Actual CO Attainment:

Faculty Analysis

Areas in which students performed well: Areas requiring improvement:

Corrective / Improvement Action:

| Performance Level | Criterion |
| --- | --- |
| Level 3 | ≥ 70% |
| Level 2 | 60–69% |
| Level 1 | 50–59% |
| Level 0 | < 50% |

Target CO Attainment:

Actual CO Attainment:

Faculty Analysis

Areas in which students performed well: Areas requiring improvement:

Corrective / Improvement Action:

Action to be implemented in the next offering:

Documents to be Maintained

The department/course faculty shall maintain:

- Assignment question/problem statement

- CO–PO and Bloom's mapping

- Assessment rubric

- Student submissions

- Tool/simulation/experimental evidence, where applicable

- Evaluated scripts/reports

- Marks and rubric-wise assessment

- CO attainment analysis

- Sample student work representing different performance levels

- Faculty analysis and continuous-improvement action